TRIBHUVAN UNIVERSITY

INSTITUTE OF ENGINEERING

KATHMANDU ENGINEERING COLLEGE

KALIMATI, KATHMANDU

Minor Project Proposal on

SurakshyaCam: AI-Based Smart Security Surveillance System

Submitted By:

Anubhav Kunwar (080BEI002)

Nischal Pandey (080BEI018)

Srijan Regmi (080BEI039)

Unik Poudel (080BEI045)

Submitted To

DEPARTMENT OF ELECTRONICS, COMMUNICATION AND INFORMATION ENGINEERING

Kalimati, kathmandu

March 16

# Acknowledgement

We would like to express our sincere gratitude to our project supervisor for their continuous guidance, valuable suggestions, and encouragement throughout the development of this minor project proposal. Their technical insights and constructive feedback greatly helped us in refining the idea, improving the system design, and completing this work in a systematic manner.

We are also grateful to the faculty members of the Department of Electronics, Communication and Information Engineering for their support, motivation, and for providing us with the necessary academic knowledge and resources required for the successful completion of this proposal.

Our sincere thanks also go to the institution for providing a supportive learning environment, laboratory facilities, and access to the tools and technologies needed to carry out this project work.

We would like to extend our appreciation to our friends and classmates for their encouragement, cooperation, and helpful discussions during the preparation of this project. Their suggestions and shared ideas helped us improve the quality of this work.

Finally, we would like to thank our family members for their constant support, patience, and motivation throughout the entire process. Their encouragement gave us the confidence to successfully complete this minor project proposal.

## Abstract

The AI-Based Smart Security Surveillance System is designed to enhance security monitoring by integrating artificial intelligence, computer vision, and embedded hardware. Traditional surveillance systems rely heavily on manual monitoring, which can be inefficient and prone to human error. This project proposes an automated solution capable of detecting suspicious activities, recognizing individuals, and generating real-time alerts.

The system utilizes an ESP32-CAM to capture live video streams, which are processed using computer vision techniques implemented in Python. The captured frames are analyzed using the YOLOv8 object detection model to identify people and potentially dangerous objects such as weapons. In addition, a face recognition module is used to classify individuals as known or unknown by comparing detected faces with a stored database. The system can also read identification cards using Tesseract OCR, allowing further verification of authorized individuals.

A PIR motion sensor connected to an Arduino Uno triggers the camera and detection process when motion is detected, helping reduce unnecessary processing and improving efficiency. When suspicious activity, unauthorized access, or dangerous objects are detected, the system sends alerts and activates warning devices such as buzzers or alarms through a relay module. All events are recorded with timestamps using a DS3231 RTC module and stored in a database for monitoring and analysis.

Furthermore, the system provides a web-based monitoring dashboard built with Django, where users can view live camera feeds, detection results, and security logs. This integrated approach combines hardware sensors, artificial intelligence, and web technologies to create a cost-effective and intelligent surveillance solution.

Overall, the proposed system improves security by enabling real-time detection, automated alerts, and centralized monitoring, making it suitable for applications such as campuses, offices, restricted areas, and smart buildings.

Table of Contents

Acknowledgement	2

Abstract	3

CHAPTER 1 - Introduction	5

1.1 Background	5

1.2 Problem Statement	6

1.3 Objectives	6

1.4 Scope	6

1.5 Applications	6

CHAPTER 2 - Literature Review	7

CHAPTER 3 - Related Theory	10

CHAPTER 4 - Feasibility Study	15

4.1 Financial Feasibility	15

4.2 Technical Feasibility	16

CHAPTER 5 - Methodology	18

5.1 System Block Diagram	18

5.2 Hardware Circuit Diagram	19

5.3 System Architecture	19

5.4 Database Schema	20

5.4 AI Processing Pipeline Architecture	20

20

5.5 IoT System Architecture	21

5.6 Gantt Chart	21

5.7 System Architecture Description	21

5.8 Algorithms	22

Entry/Exit detection using IR sensors	22

Image capture and transmission	22

Person detection using YOLOv8	22

ID card text extraction using Tesseract OCR	22

Database verification	22

Notification system	23

5.9 Hardware Design	23

Recommended hardware specifications:	23

Appendix A - GPIO Pin Mapping	23

4. Software Architecture (Detailed Design)	24

Edge Device Layer	24

AI Processing Layer	24

Cloud / Application Layer	25

7. System Workflow	25

CHAPTER 6 - Expected Results	25

8. Future Improvements	26

Appendix B - Example Codes	27

ESP32-CAM (Arduino) example	27

Python server example	28

CHAPTER 7 - References	30

# CHAPTER 1 - Introduction

## 1.1 Background

Educational institutions increasingly require automated monitoring systems that are accurate, contactless, and auditable. Conventional attendance methods such as paper registers, manual security logs, or standalone biometric devices either consume staff time or provide limited contextual information. A smart entrance monitoring system that captures both direction of movement and student identity can improve security while reducing manual work.

The rapid growth of embedded IoT hardware and lightweight vision systems now allows low-cost platforms such as the ESP32-CAM to act as event-triggered edge devices. Rather than executing all AI computation locally, the edge node can capture data and forward it to a more capable server where modern algorithms such as YOLOv8 and Tesseract OCR process images in near real time. This hybrid architecture is especially suitable for university minor projects because it balances affordability, implementability, and technical depth.

## 1.2 Problem Statement

At college entry points, security personnel often need to determine who entered, when they entered, whether they exited, and whether the individual is authorized. Manual observation is error-prone and difficult to maintain continuously. Existing low-cost camera systems record video but do not automatically understand the event, verify student identity, or update attendance records. There is therefore a need for a low-cost surveillance and attendance platform that can automatically detect entry and exit direction, capture the student and ID card image, verify the identity against a database, log the event, and notify authorized personnel when an unknown person is detected.

## 1.3 Objectives

• To design a low-cost IoT-enabled surveillance system using ESP32-CAM, IR sensors, RTC, OLED, buzzer, and relay control.

• To detect entry or exit direction by analyzing the trigger sequence of two IR sensors placed at the doorway.

• To capture event images and transmit them to a server using Wi-Fi.

• To use YOLOv8 to detect the person, face, and ID-card region from captured images.

• To use Tesseract OCR to extract the student name or ID number from the visible card.

• To verify extracted identity data against a student database and automatically record attendance.

• To alert administrators or security staff when an unknown person is detected.

• To provide a realistic academic design that can be implemented by engineering students within a minor project budget.

## 1.4 Scope

The proposed system is intended for a single entrance gate or door in a college building. It is designed to monitor movement one person at a time under moderate lighting conditions. The project scope includes local sensing, event-driven image capture, server-side AI inference, database verification, logging, and notifications. Full-scale deployment across multiple buildings, large cloud orchestration, or highly reliable legal-grade identity verification is outside the initial scope.

## 1.5 Applications

• College building entry and exit attendance monitoring

• Laboratory or library access logging

• Restricted-room monitoring for staff-only or student-only areas

• Visitor alerting and security assistance

• Prototype platform for smart campus automation

# CHAPTER 2 - Literature Review

2.1 Introduction to Smart Surveillance Systems

The rapid advancement of artificial intelligence (AI) and the Internet of Things (IoT) has revolutionized security surveillance systems. Traditional passive CCTV systems require constant human monitoring and lack instant alert mechanisms, creating significant gaps in accessibility and responsiveness (Fong, 2025). Modern intelligent surveillance systems address these limitations by automatically analyzing video feeds, detecting anomalies, and triggering appropriate responses without human intervention.

2.2 Review of Existing Systems

2.2.1 System 1: AI-Powered CCTV with IoT Alert System

Source: Fong, J. (2025). UTAR Final Year Project

This system integrates YOLOv8n for human detection, face recognition using OpenCV and Dlib, and Telegram bot alerts. The system achieved 90% face recognition accuracy and an F1 score of 0.922 for human detection, with alert delivery in 1-2 seconds. However, it lacks a web dashboard, performs poorly in low-light conditions, and has no text recognition capability.

2.2.2 System 2: IoT Motion Detection with Raspberry Pi

Source: Assauri et al. (2025). Jurnal Teknologi Informasi

A cost-effective solution using Raspberry Pi with motion detection triggering image capture and Telegram notifications with Google Drive backup. While affordable and functional, it lacks intelligent object classification, face recognition, and cannot distinguish between humans and animals.

2.2.3 System 3: Campus Facial Recognition System

Source: micpana (2025). GitHub Open Source Project

A comprehensive web-based system with Flask backend, MongoDB database, and multi-camera face recognition with threat level assignment. It features real-time facial recognition and Telegram alerts but lacks object detection, hardware integration, and OCR capabilities.

2.3 Comparative Analysis

| Feature | System 1 | System 2 | System 3 | Proposed System |
| --- | --- | --- | --- | --- |
| YOLO Object Detection | ✓ (Person only) | ✗ | ✗ | ✓ (Multiple classes) |
| Face Recognition | ✓ (90% accuracy) | ✗ | ✓ | ✓ |
| OCR/Text Recognition | ✗ | ✗ | ✗ | ✓ (Tesseract) |
| PIR Motion Sensor | ✗ | ✓ | ✗ | ✓ |
| Hardware Integration | ✓ (Arduino) | ✓ (Raspberry Pi) | ✗ | ✓ (ESP32+Arduino) |
| Physical Alarm | ✗ | ✗ | ✗ | ✓ (Relay+Buzzer) |
| Web Dashboard | ✗ | ✗ | ✓ | ✓ (Django) |
| Multi-channel Alerts | ✓ (Telegram) | ✓ (Telegram) | ✓ (Telegram) | ✓ (SMS, Email, Buzzer) |
| Database Logging | ✗ | ✗ | ✓ (MongoDB) | ✓ (SQLite+RTC) |

2.4 Research Gap Identification

The literature review reveals five significant gaps in existing systems:

Integration Gap: No system combines YOLO detection, face recognition, OCR, and hardware sensors in a unified solution.

Text Recognition Gap: OCR capability for card and text detection is absent from all reviewed systems.

Hardware-Software Gap: Systems are either software-focused or hardware-focused, lacking seamless integration where sensors trigger AI analysis and AI decisions drive hardware responses.

Comprehensive Alerting Gap: Existing systems rely on single notification channels (Telegram or email only).

Monitoring Gap: Only one system includes a web dashboard, and it lacks comprehensive real-time monitoring across all detection modalities.

2.5 Summary

This review analyzed three representative systems, identifying critical gaps that the proposed system addresses through an integrated approach combining YOLOv8 object detection, face recognition, Tesseract OCR, PIR-triggered ESP32-CAM streaming, Arduino-controlled physical alerts, multi-channel notifications, and a Django-based web dashboard with SQLite database and RTC timestamping.

## CHAPTER 3 - Related Theory

3.1 IoT Systems

Internet of Things (IoT) refers to the network of interconnected physical devices embedded with electronics, software, and sensors that enable data collection and exchange. An IoT system typically consists of four layers:

Sensing Layer: Physical devices (sensors, cameras) that capture real-world data

Edge/Gateway Layer: Intermediate processing nodes that aggregate and preprocess data

Communication Layer: Protocols (Wi-Fi, MQTT, HTTP) that transmit data between devices

Application Layer: Servers and databases that store, analyze, and present information

In the proposed project, the PIR sensor and RTC module represent the sensing layer, the ESP32-CAM acts as the edge node, Wi-Fi serves as the communication layer, and the Python server with Django dashboard forms the application layer. The main benefit of this IoT approach is that data are captured at the physical site while intelligence and storage are available remotely, enabling real-time monitoring without on-site human presence.

3.2 ESP32-CAM

The ESP32-CAM is a compact development board based on the ESP32 microcontroller with integrated Wi-Fi and Bluetooth, paired with an OV2640 image sensor capable of capturing images up to 1600×1200 resolution. Key specifications include:

Processor: ESP32 dual-core Xtensa LX6 up to 240MHz

Memory: 520KB SRAM + 4MB PSRAM

Camera: OV2640 with UXGA (1600x1200) resolution

Connectivity: 802.11 b/g/n Wi-Fi and Bluetooth 4.2

Interfaces: GPIO pins, TF card slot, flash LED

The ESP32-CAM is suitable for low-cost embedded imaging tasks because it can trigger image capture, compress frames to JPEG, and transmit them over HTTP or MQTT. In this project, it streams video when triggered by the PIR sensor via Arduino, sending frames to the laptop for AI processing.

3.3 YOLOv8 Object Detection

YOLO (You Only Look Once) is a state-of-the-art, real-time object detection algorithm that treats detection as a single regression problem, predicting bounding boxes and class probabilities directly from image pixels in one forward pass. YOLOv8, the latest iteration, offers several improvements:

Architecture: Enhanced CSPDarknet backbone with anchor-free detection

Performance: Better accuracy-speed trade-off compared to previous versions

Variants: Available in nano (n), small (s), medium (m), large (l), and x-large (x) sizes

How YOLOv8 Works:

Divides input image into a grid

Each grid cell predicts bounding boxes and confidence scores

Class probabilities are predicted for each box

Non-maximum suppression eliminates duplicate detections

In this project, YOLOv8n (nano version) is used at the server to detect persons, weapons, masks, and ID-card regions. The compact variant is appropriate for a student project as it is lighter to train (3.2M parameters) and faster to deploy on a laptop, achieving 30+ FPS on modest hardware while maintaining good accuracy.

3.4 Optical Character Recognition (OCR)

OCR is technology that converts different types of documents or images containing text into machine-readable and editable data. Tesseract OCR, originally developed by HP and now maintained by Google, is an open-source OCR engine that supports multiple languages and outputs various formats.

OCR Processing Pipeline:

| Step | Operation | Purpose |
| --- | --- | --- |
| 1 | Image Acquisition | Capture/crop card region from camera feed |
| 2 | Preprocessing | Grayscale conversion, noise reduction |
| 3 | Thresholding | Binarization for text-background separation |
| 4 | Layout Analysis | Identify text blocks, lines, words |
| 5 | Character Recognition | Match patterns against trained models |
| 6 | Post-processing | Dictionary lookup, confidence scoring |

In the proposed system, the server crops the detected card area from YOLO, converts it to grayscale, enhances contrast, applies thresholding, and sends the cleaned image to Tesseract. The extracted text is matched against known student IDs or names in the database for attendance or access verification.

3.5 Arduino UNO and Sensors

3.5.1 Arduino UNO

Arduino UNO is an open-source microcontroller board based on the ATmega328P. It features 14 digital I/O pins, 6 analog inputs, a USB connection, and operates at 16MHz. In this project, it continuously monitors the PIR sensor, triggers the ESP32-CAM upon motion detection, and activates the relay/buzzer based on commands from the Python server.

3.5.2 PIR Motion Sensor

Passive Infrared (PIR) sensors detect infrared radiation emitted by warm objects, including humans. When motion is detected, the sensor outputs a HIGH signal. PIR sensors are ideal for security applications due to their low cost, low power consumption, and reliable human detection.

3.5.3 DS3231 RTC Module

The DS3231 is a highly accurate real-time clock with integrated temperature-compensated crystal oscillator. It maintains accurate time even during power failures using a backup battery. In this project, it provides reliable timestamps for all detected events, ensuring proper logging independent of network connectivity.

3.6 Alert and Notification Systems

A notification subsystem bridges the surveillance event and the human operator. Once detection and verification are complete, the server generates appropriate alerts:

Email Alerts: Using SMTP protocol to send detailed reports with attached images

Telegram Bot: Instant messaging with real-time image delivery using Telegram Bot API

Local Buzzer: Immediate audible alarm for on-site deterrence

Web Dashboard: Real-time display of all events with filtering capabilities

For a minor project, Telegram and email are practical choices as they are simple to integrate, free to use, and require no dedicated hardware cost beyond the existing internet connection.

3.7 Web Dashboard Technologies

3.7.1 Django Framework

Django is a high-level Python web framework that encourages rapid development and clean, pragmatic design. Key features include:

ORM (Object-Relational Mapping): Simplifies database operations

Admin Interface: Automatically generated for data management

Authentication: Built-in user authentication system

Template Engine: Dynamic HTML generation

3.7.2 SQLite Database

SQLite is a lightweight, file-based relational database that requires no separate server process. It is ideal for student projects as it is simple to set up, portable, and sufficient for moderate data volumes. The database stores:

Detected event logs with timestamps

Known face encodings and associated data

System configuration parameters

3.7.3 HTML, CSS, JavaScript

These frontend technologies create an interactive dashboard showing:

Live camera feed with detection overlays

Real-time alert notifications

Historical event logs with search/filter

Known face management interface

3.8 Communication Protocols

3.8.1 Serial Communication (UART)

Used for communication between Arduino and ESP32-CAM. Simple, reliable, and suitable for trigger signals.

3.8.2 HTTP/RTSP for Video Streaming

ESP32-CAM streams video over HTTP or RTSP protocol to the Python server, enabling real-time frame processing.

3.8.3 SMS API

RESTful API that allows the Python server to send messages and to users.

# CHAPTER 4 - Feasibility Study

## 4.1 Financial Feasibility

4.1.1 Hardware Cost Estimation

| Component | Model/Specification | Unit Cost (NPR) | Quantity | Total (NPR) | Purpose |
| --- | --- | --- | --- | --- | --- |
| ESP32-CAM Module | With OV2640 camera | 1,500 | 1 | 1,500 | Video streaming and capture |
| Arduino UNO | R3 compatible | 800 | 1 | 800 | PIR sensor & relay control |
| PIR Motion Sensor | HC-SR501 | 250 | 2 | 500 | Motion detection (dual coverage) |
| DS3231 RTC Module | With backup battery | 350 | 1 | 350 | Accurate time stamping |
| Relay Module | 1-channel 5V | 200 | 1 | 200 | Buzzer/actuator control |
| Buzzer | 5V active | 100 | 1 | 100 | Local audible alarm |
| DC-DC Buck Converter | LM2596 | 300 | 1 | 300 | Voltage regulation (5V) |
| 18650 Batteries | Lithium-ion | 400 | 2 | 800 | Portable power supply |
| Battery Holder | 2-cell with wires | 150 | 1 | 150 | Battery mounting |
| Jumper Wires | Male-female set | 300 | 1 | 300 | Circuit connections |
| Breadboard | 400 points | 250 | 1 | 250 | Prototyping |
| Subtotal (Hardware) |  |  |  | 5,250 |  |
| Laptop/PC (Existing) | College lab computer | 0 | 1 | 0 | AI processing server |
| Grand Total |  |  |  | 5,250 |  |

4.1.2 Software Cost Estimation

| Software/Tool | License Type | Cost (NPR) |
| --- | --- | --- |
| Arduino IDE | Open-source | 0 |
| Python 3.x | Open-source | 0 |
| OpenCV | Open-source (BSD) | 0 |
| Ultralytics YOLOv8 | Open-source (AGPL) | 0 |
| Tesseract OCR | Open-source (Apache) | 0 |
| Django Framework | Open-source (BSD) | 0 |
| SQLite | Public Domain | 0 |
| Total Software Cost |  | 0 |

4.1.3 Cost Analysis

The total estimated project cost is approximately NPR 5,250 (approximately $40 USD), assuming an existing PC is used as the processing server. This is well within the typical budget range for academic minor projects (NPR 5,000-10,000). Key cost-saving factors include:

Reuse of existing infrastructure: College lab computer serves as AI server

Open-source software: No licensing fees for any software components

Modular design: Components can be reused for future projects

Scalable architecture: Additional cameras can be added with minimal incremental cost

Compared to commercial security systems costing NPR 30,000+ for basic CCTV with DVR, or NPR 100,000+ for AI-enabled surveillance, the proposed system offers an extremely cost-effective solution for educational institutions and small-scale deployments.

## 4.2 Technical Feasibility

The system is technically feasible because the project separates tasks according to device capability. The ESP32-CAM can reliably handle sensor reading, direction logic, image capture, Wi-Fi communication, and simple actuator control. The server performs the heavier tasks: YOLOv8 detection, OCR preprocessing, database lookup, and notifications. Each module uses mature libraries, and the complete system can be integrated with commonly available tools and programming environments.

• ESP32-CAM supports JPEG capture and HTTP client communication.

• IR sensors provide simple and inexpensive entry/exit sequence detection.

• YOLOv8n can run on a mid-range laptop CPU for prototype-scale inference, with faster performance on GPU.

• Tesseract OCR is open-source and practical for printed ID cards after preprocessing.

• SQLite or MySQL can store student data and event logs with minimal setup.

• SMS,FCM,  email SMTP, or a web dashboard can handle real-time alerts.

# CHAPTER 5 - Methodology

## 5.1 System Block Diagram

Figure 5.1: System block diagram of the proposed surveillance system.

The block diagram shows how the sensing, edge control, communication, AI processing, database, and alerting layers are connected. The ESP32-CAM is the central field node. It receives movement information from the IR sensors and time information from the RTC, captures an image, and then interacts with the OLED, buzzer, and relay based on the server decision.

## 5.2 Hardware Circuit Diagram

Figure 5.2: Simplified hardware circuit diagram with major signal and power connections.

# 5.3 System Architecture

# 5.4 Database Schema

# 5.4 AI Processing Pipeline Architecture

Figure 5.4: AI processing pipeline used at the server.

## 5.5 IoT System Architecture

Figure 5.5: Layered IoT system architecture.

## 5.6 Gantt Chart

Figure 5.6: Proposed eight-month project schedule.

## 5.7 System Architecture Description

The overall architecture is divided into three logical layers. The edge device layer consists of the ESP32-CAM, IR sensors, RTC, OLED, buzzer, and relay. The AI processing layer consists of a Python server running image preprocessing, YOLOv8 inference, OCR, and event classification. The cloud or application layer consists of the student database, event log, notification engine, and dashboard. This partitioning keeps hardware cost low while still enabling advanced AI.

## 5.8 Algorithms

### Entry/Exit detection using IR sensors

1. Wait until any IR sensor changes from idle to active.

2. If IR1 is triggered before IR2 within a valid timing window, classify as Entry.

3. If IR2 is triggered before IR1 within a valid timing window, classify as Exit.

4. Ignore incomplete sequences and apply a debounce timer to avoid repeated triggers from the same person.

5. Store the classified direction as metadata attached to the captured image.

### Image capture and transmission

1. On valid direction event, trigger ESP32-CAM frame capture.

2. Compress the frame as JPEG to reduce transfer size.

3. Read current date and time from DS3231 RTC or NTP backup.

4. Build a multipart HTTP POST or binary upload packet containing image, timestamp, and direction.

5. Send the packet to the server endpoint and wait for acknowledgment.

### Person detection using YOLOv8

1. Receive image at server.

2. Resize image to detector input size and normalize pixel values.

3. Run YOLOv8 inference to detect person, face, and optionally card.

4. Reject image if no person is detected.

5. Pass face/card regions to the next stage for OCR preprocessing.

### ID card text extraction using Tesseract OCR

1. Crop probable ID-card region.

2. Convert to grayscale and apply contrast enhancement.

3. Use thresholding, denoising, and perspective correction where needed.

4. Run Tesseract OCR with suitable page segmentation mode.

5. Post-process text using regex patterns and string cleaning to isolate student ID or name.

### Database verification

1. Search exact or fuzzy match of extracted ID in the student table.

2. If a valid record exists, classify as Authorized Student.

3. If no valid record exists, classify as Unknown Person.

4. Create an attendance/event record containing timestamp, direction, extracted text, and image path.

### Notification system

1. If Authorized Student, optionally send a normal attendance notification or only log silently.

2. If Unknown Person, send alert message with image to security/admin.

3. Update dashboard, buzzer state, and optional relay action according to event type.

4. Archive event data for audit and future analysis.

## 5.9 Hardware Design

The hardware node is designed around one ESP32-CAM module installed near the entrance. Two IR sensors are fixed on the door frame at a small spacing to determine movement direction. The DS3231 RTC provides accurate timestamp backup. The OLED gives immediate local status such as 'Entry Logged' or 'Unknown Person'. The buzzer gives brief audio feedback. A relay module can energize a door lock or drive a motor control stage for a gate.

### Recommended hardware specifications:

• ESP32-CAM: AI Thinker ESP32-CAM with OV2640 camera, 4 MB PSRAM preferred.

• IR sensors: 2 x infrared obstacle sensors or break-beam sensors, 5 V supply.

• RTC: DS3231 I2C module with coin-cell backup.

• OLED: 0.96 inch SSD1306 I2C display, 128 x 64 pixels.

• Buzzer: 5 V active buzzer or transistor-driven passive buzzer.

• Relay: 1-channel 5 V opto-isolated relay module.

• Door lock: 12 V solenoid lock or SG90/MG996R servo depending on gate mechanism.

• Power: 5 V / 2 A regulated supply for node electronics; separate higher-current supply for lock if used.

# Appendix A - GPIO Pin Mapping

| ESP32-CAM Pin | Function | Connected Module | Notes |
| --- | --- | --- | --- |
| 5V | Power input | 5 V regulated supply | Use stable 2 A source |
| GND | Ground | All modules | Common reference ground |
| GPIO13 | Digital input | IR Sensor 1 output | Entry/exit sequence input |
| GPIO12 | Digital input | IR Sensor 2 output | Entry/exit sequence input |
| GPIO14 | Digital output | Relay IN | Controls lock or gate driver |
| GPIO15 | Digital output | Buzzer | Short beep on result |
| GPIO2 | I2C SDA | OLED + DS3231 | Shared I2C bus |
| GPIO4 | I2C SCL | OLED + DS3231 | Shared I2C bus |
| UART U0R/U0T | Programming | FTDI programmer | Used during firmware upload |
| OV2640 interface | Camera | Built-in | Reserved for camera signals |

# 4. Software Architecture (Detailed Design)

## Edge Device Layer

• Initialize camera, Wi-Fi stack, RTC, OLED, buzzer, and GPIOs.

• Continuously sample IR sensors with debounce logic.

• Classify movement direction using sensor sequence timing.

• Capture JPEG image after valid trigger.

• Package metadata: timestamp, direction, device ID, event sequence number.

• Upload image and metadata to server endpoint.

• Receive response and actuate buzzer, OLED, and optional relay.

## AI Processing Layer

• Server receives image through REST API.

• OpenCV preprocessing resizes, denoises, and optionally corrects perspective.

• YOLOv8 validates the presence of a person and localizes card region.

• Tesseract OCR extracts the printed ID or name.

• String cleaning and pattern matching improve OCR reliability.

• Business logic determines whether the event is authorized or unknown.

## Cloud / Application Layer

• Student database stores enrollment ID, name, department, and permitted access fields.

• Event log table stores time, direction, extracted text, authorization result, and image path.

• Notification module sends messages to admin or security.

• Dashboard provides query, filtering, and image review interface.

• Attendance export can be generated as CSV or PDF.

# 7. System Workflow

1. A student approaches the entrance gate and interrupts one IR sensor.

2. The second IR sensor triggers shortly afterward, allowing the ESP32-CAM to classify the movement as entry or exit based on sequence order.

3. The ESP32-CAM reads the RTC timestamp and captures a JPEG image containing the person and student card.

4. The image and metadata are transmitted over Wi-Fi to the processing server.

5. The server preprocesses the image and runs YOLOv8 to confirm the presence of a person and optionally isolate the visible ID-card region.

6. The cropped card image is passed to Tesseract OCR for text extraction.

7. The extracted student ID or name is cleaned and compared with the student database.

8. If a valid match is found, the system records an attendance event with direction and time, updates the OLED display, and optionally unlocks the door briefly.

9. If no valid match is found, the system stores the image as an alert event and sends a security notification with the event details.

10. All events remain available in the dashboard and database for later review.

# CHAPTER 6 - Expected Results

• Automatic entry and exit logging for authorized students.

• Accurate timestamping of events using RTC-backed time records.

• Near real-time notifications for unknown or unverified persons.

• Improved campus security through image evidence and database-backed logging.

• Reduced manual attendance effort at monitored doors.

• A reusable academic platform for future smart campus research.

The expected operational result is a working prototype capable of monitoring one controlled doorway under indoor lighting with moderate traffic. System performance will depend on camera placement, ID-card visibility, network strength, and OCR quality. In practice, multiple test iterations will be required to tune sensor spacing, image framing, and OCR preprocessing.

# 8. Future Improvements

• Integrate face recognition after obtaining necessary privacy approval.

• Support multi-camera deployment using a central campus server.

• Move some inference tasks to edge AI hardware such as ESP32-S3 or NVIDIA Jetson.

• Add mobile app monitoring for security officers.

• Introduce anti-tailgating logic for multiple people entering together.

• Integrate with smart campus ERP or academic attendance systems.

• Add dashboard analytics such as peak-hour counts, frequent unknown alerts, and access

trends.

# Appendix B - Example Codes

## ESP32-CAM (Arduino) example

#include <WiFi.h>

#include "esp_camera.h"

// Wi-Fi credentials

const char* ssid = "_WIFI";

const char* password = "PASSWORD";

// Server IP (laptop running Python)

const char* serverIP = "192.168.1.100";

const int serverPort = 5000;

// PIR trigger pin from Arduino

const int triggerPin = 13;

bool motionDetected = false;

// Camera model

#define CAMERA_MODEL_AI_THINKER

#include "camera_pins.h"

void setup() {

Serial.begin(115200);

pinMode(triggerPin, INPUT);

// Connect to Wi-Fi

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

}

// Initialize camera

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM;

// ... (other pin configs)

config.frame_size = FRAMESIZE_QVGA;

config.jpeg_quality = 12;

esp_camera_init(&config);

}

void loop() {

// Check if triggered by Arduino (PIR motion)

if (digitalRead(triggerPin) == HIGH || motionDetected) {

motionDetected = true;

captureAndSend();

delay(1000); // Cooldown

motionDetected = false;

}

}

void captureAndSend() {

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) return;

// Send to Python server

WiFiClient client;

if (client.connect(serverIP, serverPort)) {

client.println("POST /detect HTTP/1.1");

client.println("Content-Type: image/jpeg");

client.print("Content-Length: ");

client.println(fb->len);

client.println();

client.write(fb->buf, fb->len);

}

esp_camera_fb_return(fb);

}

## Python server example

import cv2

import numpy as np

from django.shortcuts import render

from django.http import JsonResponse, StreamingHttpResponse

from django.views.decorators.csrf import csrf_exempt

from .models import Event

from ultralytics import YOLO

import face_recognition

import pytesseract

# Load AI models

yolo = YOLO('yolov8n.pt')

known_faces = []  # Load from DB in real project

def dashboard(request):

events = Event.objects.all()[:20]

return render(request, 'dashboard.html', {'events': events})

def live_feed(request):

def gen():

while True:

frame = cv2.VideoCapture(0).read()[1]  # Simplified

_, jpeg = cv2.imencode('.jpg', frame)

yield (b'--frame\r\nContent-Type: image/jpeg\r\n\r\n' +

jpeg.tobytes() + b'\r\n')

return StreamingHttpResponse(gen(), content_type='multipart/x-mixed-replace; boundary=frame')

@csrf_exempt

def detect_api(request):

if request.method == 'POST':

# Get image

img = cv2.imdecode(np.frombuffer(request.body, np.uint8), cv2.IMREAD_COLOR)

# YOLO detection

results = yolo(img)

for r in results:

for box in r.boxes:

conf = float(box.conf[0])

cls = r.names[int(box.cls[0])]

# Simple processing

if cls == 'person':

# Face recognition

x1, y1, x2, y2 = map(int, box.xyxy[0])

face = img[y1:y2, x1:x2]

rgb = cv2.cvtColor(face, cv2.COLOR_BGR2RGB)

enc = face_recognition.face_encodings(rgb)

if enc and face_recognition.compare_faces(known_faces, enc[0]):

Event.objects.create(event_type='known', confidence=conf)

else:

Event.objects.create(event_type='unknown', confidence=conf)

elif 'card' in cls:

# OCR

x1, y1, x2, y2 = map(int, box.xyxy[0])

card = img[y1:y2, x1:x2]

gray = cv2.cvtColor(card, cv2.COLOR_BGR2GRAY)

text = pytesseract.image_to_string(gray)

Event.objects.create(event_type='card', details=text, confidence=conf)

return JsonResponse({'status': 'ok'})

def send_sms(msg):

import requests

requests.post("https://smsprovider.com/api/send", json={

'api_key': 'your_key',

'to': '98XXXXXXXX',

'message': msg

})

def events_api(request):

events = Event.objects.values()[:50]

return JsonResponse(list(events), safe=False)

# CHAPTER 7 - References

• J. Redmon et al., works on real-time object detection and later YOLO-family developments.

• Ultralytics documentation for YOLOv8 deployment and inference workflows.

• R. Smith, 'An Overview of the Tesseract OCR Engine,' ICDAR.

• Espressif Systems, ESP32 and ESP32-CAM related hardware documentation.

• DS3231 RTC module and SSD1306 OLED module technical datasheets.

• OpenCV and Flask official documentation for image processing and API development.

Redmon, J., et al., 'You Only Look Once: Unified Real-Time Object Detection', IEEE CVPR.
[2] Ultralytics, 'YOLOv8 Documentation', Ultralytics.
[3] Smith, R., 'An Overview of the Tesseract OCR Engine', IEEE.
[4] Espressif Systems, 'ESP32-CAM Technical Reference Manual'.
[5] Tanenbaum, A., 'Computer Networks', Pearson.
[6] Rajkumar Buyya, 'Internet of Things: Principles and Paradigms', Elsevier.
[7] Goodfellow, I., Bengio, Y., Courville, A., 'Deep Learning', MIT Press.